# CWMS Location and Time Series Groups

Contents

Overview	1

Organization	1

Structure	1

Category Structure	1

Group Structure	1

Group Membership Structure	2

Using Aliases	2

Uniqueness	3

Partially-Aliased Identifiers	3

Naming Standards	4

Views	4

# Overview

The CWMS database supports collecting locations and time series items into groups of related items.  The nature of the relation for any group is defined by the user. The database structure imposes no semantic requirements on the groups and provides a variety of fields to support various uses.  Location groups and time series groups are nearly identical in their structure and usage and are treated together in this document, with the few differences specified in the appropriate sections.

# Organization

Grouping uses a two tier hierarchy.  The top tier is the category and the bottom tier is the group.  Each category can contain many groups, while each group belongs to only one category.  Each group may contain many locations or time series, and locations and time series may be assigned to many groups.

Each category and each group is owned by a single office in the CWMS database.  Each office can use the CWMS database API to manage their categories and groups programmatically or use the CWMS Management Application (CMA) to do so interactively.  Groups can belong only to categories owned by the same office that owns the group or by the CWMS pseudo-office.  Locations and time series can be assigned only to groups owned by the same office that owns the location or time series or by the CWMS pseudo-office.  No category or group owned by the CWMS pseudo-office may be deleted or renamed by any office either programmatically or via CMA.

# Structure

## Category Structure

Categories contain only enough information to identify and describe them, and to relate them to offices and groups.  This information is contained in the tables AT_LOC_CATEGORY and AT_TS_CATEGORY.

- Office Code: a numeric value that relates the category to the owning office.

- Category Code: a numeric value that relates the category to the groups it contains.

- Category ID: a string of up to 32 characters that identifies the category.  This identifier is case-retained; it may be located using any character case, but the original character case id retained.

- Category Description: a string of up to 256 characters that describes the category.

## Group Structure

Groups contain information similar to categories plus some additional items.  This information is contained in the tables AT_LOC_GROUP and AT_TS_GROUP.

- Office Code: a numeric value that relates the group to the owning office.

- Category Code a numeric value that relates the group to the category that contains it.

- Group Code: a numeric value that relates the group to the locations or time series it contains.

- Group ID: a string of up to 65 characters that identifies the group.  This identifier is case-retained; it may be located using any character case, but the original character case id retained.

- Group Description: a string of up to 256 characters that describes the group.

- Shared Alias ID: an optional text string of up to 256 characters that can be used as a many-to-one alias for the contained locations or time series.

- Shared Reference Code: an optional numeric value that can be used to relate all contained locations or time series to a single location or time series.  Location groups may reference only locations and time series groups may reference only time series.

## Group Membership Structure

Group membership contains information to relate a location or time series to a group as well as information that may be used to differentiate the members within the group or apply to the location or time series as a result of group membership.  This information is contained in the tables AT_LOC_GROUP_ASSIGNMENT and AT_TS_GROUP_ASSIGNMENT.

- Location or Time Series Code: a numeric value that relates the membership to the location or time series.

- Office Code: a numeric value that relates the location or time series to its owning office.  Although this information is implicit in the location or time series code, it is also explicitly specified to facilitate indexed lookups of locations or time series by office and alias ID.

- Group Code: a numeric value that relates the membership to the group.

- Attribute: an optional numeric value that may be used to differentiate the members of the group by some user-defined measure.  A common use is to provide membership order for groups in which order is meaningful.

- Alias ID: an optional string of up to 256 characters that is used as a one-to-one alias for the location or time series as a result of membership in the group.

- Reference Code: an optional numeric value that may be used to relate the location or time series to another location or time series as a result of membership in the group.  Location group memberships may reference only locations and time series group memberships may reference only time series.

# Using Aliases

One of the more common uses of groups within the CWMS database is to provide aliases for locations or time series.  The CWMS database provides a CWMS-owned location category named “Agency Aliases” which contains the following CWMS-owned groups, all expressly for the purpose of providing location aliases:

- CBT Station ID

- DCP Platform ID

- NRCS Station ID

- NWS Handbook 5 ID

- SHEF Location ID

- TVA Station ID

- USBR Station ID

- USGS Station Name

- USGS Station Number

## Uniqueness

Aliases must be unique within groups.  That is, no location or time series group may contain different locations or time series with the same alias.

The same location alias may be used in multiple location groups only if the same location is aliased in each instance.  Attempting to use an existing location alias for a location other than the one it is already used for will cause the database to raise an exception and abort the operation.  The exception text is:

ERROR: Alias (<alias_id>) would reference multiple locations.  If you want to allow this, set the CWMSDB/Allow_multiple_locations_for_alias property to 'T' for office id <office_id>.  Note that this action will eliminate the ability to look up a location using the alias or any others that reference multiple locations.

Note that the text specifies that an office may relax the requirement that an alias identifier resolve to a single location for its office.  If the need arises to have multiple locations share a single alias, consider creating a location group for that purpose and using the shared alias identifier before relaxing the requirement.

While time series aliases must be unique within time series groups, the database does not currently enforce a globally unique time series alias to time series relationship as it does for locations.

## Partially-Aliased Identifiers

Since compound identifiers in the CWMS database normally contain location identifiers as one of their components, location aliases can allow the compound identifiers for the aliased location to be identified by text comprised of the normal compound id with the location portion replaced with the location alias.

For example, suppose there is a location named “KeystoneDam” with aliases “KEYO2”, “07164200 “, and “CE609B50”; and a time series named “KeystoneDam.Elev.Inst.1Hour.0.Goes-rev”.  Because of the location aliases, all of the following may be used to access the same time series:

- KeystoneDam.Elev.Inst.1Hour.0.Goes-rev

- KEYO2.Elev.Inst.1Hour.0.Goes-rev

- 07164200.Elev.Inst.1Hour.0.Goes-rev

- CE609B50.Elev.Inst.1Hour.0.Goes-rev

Further suppose that “KeystoneDam” has a sub-location named “SluiceGate1” with an associated time series name “KeystoneDam-SluiceGate1.Opening.Inst.0.0.Manual-rev”.  Although only the base location portion of the time series identifier is aliased, all of the following may be used to access the same time series:

- KeystoneDam-SluiceGate1.Opening.Inst.0.0.Manual-rev

- KEYO2-SluiceGate1.Opening.Inst.0.0.Manual-rev

- 07164200-SluiceGate1.Opening.Inst.0.0.Manual-rev

- CE609B50-SluiceGate1.Opening.Inst.0.0.Manual-rev

Note that these are not time series aliases in the sense that they are aliases specified in a time series group, but are a by-product of the fact that the time series identifier includes a location identifier that is aliased.

## Naming Standards

Locations and time series both have requirements on the text format of their respective identifiers.  Location and time series aliases are not required to conform to those same formats, but may contain any text up to 256 characters in length.  Be aware that while this capability may be desirable in some contexts, using aliases that do not conform to the format requirements of the aliased identifiers may cause the aliases to be unavailable or otherwise unusable in software that expects the required formats.

## Views

Some views in the CWMS database have been modified to include location and/or time series aliases, while other views have been added for this purpose.  Views that contain aliases will have multiple rows that contain the same location code or time series code, which is a departure from views that don’t contain aliases.  The modified or new views have the following columns to help identify aliases.

| Column Name | Values | Comments |
| --- | --- | --- |
| ALIASED_ITEM | BASE LOCATION | Identifier is partially aliased; the base location is aliased. |
|  | LOCATION | If identifier is a location, it is aliased. |
|  |  | If identifier is compound, it is partially aliased; the location portion is aliased. |
|  | TIME SERIES* | Time series identifier is aliased. |
| LOC_ALIAS_CATEGORY | <Loc Category ID> | Null if location or base location is not aliased. |
| LOC_ALIAS_GROUP | <Loc Group ID> | Null if location or base location is not aliased. |
| TS_ALIAS_CATEGORY* | <TS Category ID> | Null if time series is not aliased. |
| TS_ALIAS_GROUP* | <TS Group ID> | Null if time series is not aliased. |

*Applicable only in views that contain time series identifiers.

The views that contain aliases are:

| View Name | Status | Column Containing Alias |
| --- | --- | --- |
| CWMS_V_CWMS_TS_ID2 | New | CWMS_TS_ID |
| CWMS_V_LOC2 | New | LOCATION_ID |
| CWMS_V_RATING | Updated | RATING_ID |
| CWMS_V_RATING_LOCAL | Updated | RATING_ID |
| CWMS_V_RATING_SPEC | Updated | RATING_ID |
| CWMS_V_TSV_DQU | Updated | CWMS_TS_ID |

In these views, use the ALIASED_ITEM column to interpret the contents of the column containing the alias.  Note that other columns will not be modified, so from the example above with a sub-location and an aliased base location, the CWMS_V_LOC2 view would contain the following:

| BASE_LOCATION_ID | SUB_LOCATION_ID | LOCATION_ID | ALIASED_ITEM |
| --- | --- | --- | --- |
| KeystoneDam | SluiceGate1 | KeystoneDam-SluiceGate1 |  |
| KeystoneDam | SluiceGate1 | KEYO2-SluiceGate1 | BASE LOCATION |
| KeystoneDam | SluiceGate1 | 07164200-SluiceGate1 | BASE LOCATION |
| KeystoneDam | SluiceGate1 | CE609B50-SluiceGate1 | BASE LOCATION |

To use the new or updated views to retrieve only non-aliased identifiers, add the text “AND ALIAISED_ITEM IS NULL” to the WHERE clause in your query.  If the query doesn’t have a WHERE clause, add the text “WHERE ALIASED_ITEM IS NULL” to create one.